Comparative Study of Internal Force in Lecture Buildings in DIY between SNI 1726-2012 and 1726-2019 with the Spectrum Response Method (2D)

Shilmi Arifah1, Slamet Widodo2, Daniel Jesayanto Jaya3

1,3Postgraduate Program of Technology and Vocational Education, Yogyakarta State University, Yogyakarta, Indonesia

2Departement of Civil Engineering Education, Faculty of Engineering, Yogyakarta State University, Yogyakarta, Indonesia

Email: shilmiarifah.2019@student.uny.ac.id

Abstract. An earthquake is a sudden movement of the ground that comes from a wave in a place and spreads from the area in all directions. In 2019 the Government of Indonesia has ratified a new standard on procedures for earthquake resistance planning for building and non-building structures, namely SNI 1726: 2019 as a revision of SNI 1726: 2012. The adoption of SNI 1726-2019 as a new standard has added and changed the scope of the procedure for calculating earthquakes to become wider so that it can follow the current developments. Earthquake loads can be analyzed both statically and dynamically. Dynamic analysis used the response spectrum method. This study aims to compare the results of the calculation using the response spectrum using SNI 1726-2012 and SNI 1726-2019 in order to find out whether buildings using the old rules are still safe or not. The building structure is modeled on a regular building and functioned as a lecture building in the Province of D.I Yogyakarta with a building height of 14.10 m. The results of the research prove that the structure of the lecture building which still uses the SNI 1726-2012 method must be rehabilitated so that it is able to withstand earthquake loads based on the new SNI 1726-2019 method. This occurs due to changes in the earthquake map due to the discovery of new faults in Indonesia.

- Introduction

An earthquake is a sudden movement of the ground that comes from a wave in a place and spreads from the area in all directions. Naturally, earthquakes that occur are caused by the release of energy in the earth that has been trapped long enough to create waves in all directions (Pratiwi & Widodo, 2012) (Bayyinah & Faimun, 2017). Earthquakes shake the earth which results in a sudden release of energy which creates seismic waves (Dadawala & N. R., 2018). Earthquakes in relation to a region are related to the movement of the earth's surface and its effect on the area concerned.

Indonesia is a country that has the potential to experience an earthquake. This is because Indonesia is in the area of ​​the Pacific earthquake route (Circum Pacific Earthquake Belt) and the Asian earthquake route (Trans Asiatic Earthquake Belt) (Pratiwi & Widodo, 2012). In planning development in Indonesia, earthquake parameters must be considered. One of the earthquake parameters that directly affects the planning is the ground acceleration caused by seismic waves acting on the building period. In order to secure the building, the building must be designed as an earthquake resistant building which is planned in accordance with the applicable earthquake regulations in Indonesia. The collapse of buildings can be minimized by analyzing building modeling during an earthquake so that planners can better plan the buildings to be built (Handayani, 2015).

In 2019 the Government of Indonesia has ratified a new standard on procedures for earthquake resistance planning for building and non-building structures, namely SNI 1726: 2019 as a revision of SNI 1726: 2012. It is stated in the standard that the effect of an earthquake plan must be reviewed in the planning and evaluation of building and non-building structures (Faizah, 2015). The design seismic force can be calculated using the equivalent lateral force analysis, analysis of the variance response spectrum as well as seismic time history response analysis.

Dynamic analysis for the design of earthquake-resistant structures is carried out if a more accurate evaluation of the seismic forces acting on the structure is needed, and to determine the behavior of the structure due to the effects of the earthquake (W, 2015). In the research (Suntoko, Marwanto, Mp, & Iswanto, 2019) the results of dynamic analysis of the response spectrum using earthquake plans based on SNI 1726-2012, found that the building has a value of S1 0.30g and Ss 0.75g. Analysis of the soil compression on the basement wall per m2 is 27.361 kN / m2 or 0.2.736 t / m2 The period of vibration is 0.0926 seconds, the seismic response coefficient is 0.1800, the distribution of lateral forces is 578.489 tonnes. -Every floor level so that when reviewed based on ATC-40 it is included in the Immediate Occupancy level category. It is reinforced by research (A, 2017) that comparisons using response spectrum analysis and time history analysis on a 4-story structure have the results of displacement and drift values ​​that are not much different, with a value of 0.053919 m in the response spectrum and 0.052729 m in the time history for displacement value, while the drift value is 0.003866 m in the response spectrum and 0.003723 m in the time history. So that the building is safe against service-free {(0.03 / R) x h}, ultimate limit (0.02h), and ATC-40 (Immediate Occupancy).

With so many building plans in Indonesia that still use SNI 1726-2012 with the dynamic response spectrum analysis method. For this reason, it is necessary to conduct an evaluation of the existing building planning and then compare the capacity of the building in facing the earthquake strength defined in SNI 1726-2012 and SNI 1726-2019 by comparing the values ​​of Mu, Pu, and Vu and minimizing the number of casualties when an earthquake occurs.

- Loadings

In this study two types of load are used, i) gravity load which includes dead load and live load, and ii) lateral load which includes earthquake loads.

## Gravity load

Table 1. Type of Load

This loading represents all tributary dead loads and live loads. The live load and dead load based on PPIUG 1983 & PPURG 1987 refer to SNI 2847-2013. The dead load and live load work as distributed loads on buildings.

| Type of Load | kN/m2 |
| --- | --- |
| Dead load works on the floor | 4,54 |
| Dead load works on the beam | 2,5 |
| Dead load works on the roof | 3,12 |
| Live load works on the floor | 1,92 |
| Live load works on the roof | 1 |

## Lateral load

In lateral load, there are various types of load, but the focus of this research is earthquake load using the spectrum response method between SNI 1726-2012 and SNI 1726-2019

    - Design Spectrum Responses

The design spectrum response that will be used in the analysis comparison is SNI 1726-2012 and SNI 1726-2019. The design response spectrum is required to take into account the influence of torque, moment and displacement of the center of mass from the center of rigidity. The stages of the spectrum response design according to SNI 1726-2012 and 2019 in a way that has been adjusted to the research needs (Dwi Sungkono, 2019) :

- Observe the location of the building to be evaluated

- Determine SPT value and classification of soil type

- Determine the parameters of the acceleration of the maximum earthquake response with consideration of the risk targeted MCER (Maximum Considered Earthquake, Risk Targeted) for short periods of 0.2 seconds (SS) and 1.0 seconds (S1).

- Look for the amplication factor of the response parameters of the ground surface acceleration spectrum for short periods (Fa) and 1 second period (Fy).

- Determine the parameters of the acceleration spectrum response (SMS and SM1),

SMS = Fa x Ss

SM1 = Fy x S1

- Determine the parameters of the design acceleration spectrum response (SDS and SD1),

SDS = 2/3 x SMS

SD1= 2/3 x SM1

- Make a graph response to images with the provisions that have been determined SNI 1726-2012

Conditions:

- For periods smaller than T0, the design acceleration response spectrum, Sa, must be taken from Equation Sa = SDS (0,4 + 0,6 (T/T0))

- For periods greater than or equal to T0 and smaller than or equal to Ts the response spectrum of the design Sa = SDS.

- For periods greater than Ts, the design acceleration response spectrum, Sa, is taken for equation: Sa = SD1/T

- Make a graph response to images with the provisions that have been determined SNI 1726-2019.

Period provisions are as follows:

- For periods smaller than T0, the following equation is used Sa = SDS (0,4 + 0,6 T/T0)

- For periods greater than or equal to T0 and smaller than or equal to TS, the response spectrum of the design Sa = SDS

- For periods greater than TS, but smaller than or equal to TL, the spectral response of the design acceleration, Sa, is taken equation Sa = SD1/T

- For periods greater than TL, the spectral response of the design acceleration, Sa, is taken based on Equation Sa = (SD1 x TL)/ T2

- For the value of T obtained by calculating the fundamental period of the structure (T) based on this equation:

Ta = Ct. hnX

hX = the height of the structure (m) above the base to the highest level of the structure, Ct and x are obtained from the table specified by SNI 1726-2012 and SNI 1726-2019

- For the values of T0 and TS can be obtained from the equation:

T0 = 0,2 x SD1/SDS

TS = SD1/SDS

- For TL values, obtained from Figure 20 in SNI 1726-2019.

- In determining the risk category of buildings and earthquake priority factors, it is found that the lecture buildings is categorized as IV and the earthquake priority factor is 1.50

- Determination of seismic design categories is based on the seismic design category of the acceleration response parameters in the short and 1 second periods

  - Internal Force

Internal force is the force that holds the propagation force in the construction to achieve balance. Internal forces include axial forces, latitude forces, and moments (Rohman, 2017). The results that will be obtained from the internal force are the Ultimate Moment (Mu), the Ultimate Shear Strength (Vu), and the Ultimate Pressure (Pu)

- Method

This study aims to check and evaluate the change in spectrum response in lecture buildings that are still using planning with SNI 1726-2012 and compared with SNI 1726-2019, after counting the response spectrum is followed by running model in SAP with the Dynamic method (Response Spectrum) ) then compared the values of internal force such as Mu, Pu, and Vu. Modeling in 2D in SAP.

Figure 1. Flow chart diagram

## Preliminary Data

Preliminary data includes general soil data, structural technical data, and earthquake data.

    - Soil Data.

- The lecture building consists of 4 floors with an area of 17 m x 57 m. The N-SPT data is obtained through the assumptions given by (Safwan, 2018) using the CPT and SPT methods.Structural Technical Data.

- Lecture buildings withstand structural loads based on Indonesian government standards using SNI 2847-2013. The lecture buildings uses the Reinforcement concrete structure, the foundation uses a cakar ayam. The concrete quality (fc’) used is 25 MPa, the steel quality (fy) is 240 MPa and 400 MPa.

Table 2. Beam, Colomn, and Sloof Size

| Type of Beam | Size (cm) | Type of Column | Size (cm) | Type of Sloof | Size (cm) |
| --- | --- | --- | --- | --- | --- |
| B1 | 35 x 70 | C1 | 40 x 70 | Ps | 10 x 20 |
| B2 | 30 x 40 | C1a | 40 x 40 |
| B2a | 30 x 40 | C2 | 40 x 40 |
| B3 | 30 x 50 | C3 | 20 x 40 x 40 |
| B3a | 30 x 50 | C4 | 25 x 40 x 40 |
| B4 | 30 x 40 | C5 | 30 x 30 |
| B5 | 20 x 40 | C6 | 30 x 30 |
| B5a | 20 x 40 | C7 | 35 x 60 |
| B5b | 20 x 40 | C8 | 30 x 35 |
| B5c | 25 x 40 | C9 | 20 x 25 |
| B5d | 20 x 40 | PC | 10 X 10 |
| B5e | 20 x 40 | PC2 | 10 X 30 |
| B6 | 30 x 50 | PC3 | 10 X 40 |
| BS | 25 x 40 | PC4 | 15 X 45 |
| TB1 | 30 x 50 |
| TB2 | 25 x 40 |
| L1 | 12 x 26 |
| L2 | 12 x 65 |
| LB01 | 10 X 10 |
| LB02 | 24 X 10 |
| LB03 | 10 X 30 |
| PB | - 20 |

    - Earthquake data. The spectrum response value is obtained by following the predetermined stages:

- The location of the building being reviewed is in DIY.

- With an SPT value of 18, then determine the class of the land site using the Site Classification table, obtained medium soil (SD)

Table 3. The parameter of acceleration of spectrum response earthquake is based on SNI 1726-2012 and SNI 1726-2019

Determination of parameters for earthquake response acceleration based on SNI 1726-2012 and SNI 1726-2019

| Parameters | SNI 1726-2012 | SNI 1726-2019 |
| --- | --- | --- |
| Ss | 0,75 | 0,75 |
| S1 | 0,3 | 0,3 |
| Fa | 1,2 | 1,2 |
| Fv | 1,8 | 2,0 |
| SMS | 0,9 | 0,9 |
| SM1 | 0,54 | 0,6 |

- Result and Discussion

## Loading.

## Loading the structure of the lecture buildings using a standard combination of structural loads SNI 2847-2013. Standard response of earthquake response spectrum using SNI 1726-2012 and SNI 1726-2019.

## Structure Analysis.

Analysis of lecture buildings structure using computer software, SAP 2000. Part to be reviewed is selected the longest transverse side to be given evenly distributed force and if it is drawn it will be like the picture below

Figure 2. Sketch of lecture buildings

## Calculation of Earthquake Loading.

For earthquake loading we use response spectrum method between SNI 1726-2012 and SNI 1726-2019. From the initial spectrum response data, an acceleration calculation is performed to obtain the spectrum response spectrum using two different standards, namely SNI 1726-2012 and SNI 1726-2019.

    - SNI 1726-2012 and SNI 1726-2019.

- Spectrum response graphs are obtained based on the spectrum response method specified in SNI 1726-2012 and SNI 1726-2019, the values obtained are then entered into SAP software to get the spectrum response graphs used to determine earthquake resistance of buildings.

Table 4. The value used to make response spectrum graph based on SNI 1726-2012

| Respons Spektrum dengan SNI 1726-2012 |  | Respons Spektrum dengan SNI 1726-2019 |  |
| --- | --- | --- | --- |
| SD1 | 0,36 | SD1 | 0,4 |
| SDs | 0,6 | SDs | 0,6 |
| T0 | 0,12 | T0 | 0,133 |
| Ts | 0,6 | Ts | 0,667 |
| Sa = SDS (0,4+0,6 x T/T0) | 1,752 | Sa = SDS (0,4+0,6 x T/T0) | 1,604 |
| Sa=SDS | 0,6 | Sa=SDS | 0,816 |
| Sa = SD1/T | 0,714 | Sa = SD1/T | 0,794 |
| T= | 0,504 | T= | 0,504 |
|  |  | TL = | 20 |

From the SNI 1726-2012 and SNI 1726-2019 methods, the spectrum response graphs in figures 3 and 4 are obtained.

Sa = SDS

Sa = SDS ( 0.4 + 0.6. T/T0)

Sa = SD1/T

Figure 3. SNI 1726-2012 response spectrum graph

Sa = SD1/T

Sa = (SD1/TL )/T2

Sa = SDS ( 0.4 + 0.6. T/T0)

Sa = SDS

Figure 4. SNI 1726-2019 response spectrum graph

With static load and spectrum response of SNI 1726-2012 and SNI 1726-2019 which have been defined, SAP2000 calculation can be done. Then after the SAP2000 results table is obtained, then compare the values of Mu, Pu, Vu.

Table 5. Comparison of internal force between SNI 1726-2012 and SNI 1726-2019

| Element | Pu |  | Vu |  | Mu |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SNI-1726-2012 | SNI 1726-2019 | SNI-1726-2012 | SNI 1726-2019 | SNI-1726-2012 | SNI 1726-2019 |
| 1 | 690,763 | 686,537 | 70,657 | 67,089 | 172,1175 | 169,0223 |
| 2 | 379,035 | 376,789 | 117,948 | 115,218 | 240,4284 | 234,3186 |
| 3 | 68,031 | 67,318 | 56,025 | 55,246 | 159,2193 | 159,3401 |
| 4 | 905,821 | 905,824 | 65,656 | 61,599 | 153,2118 | 148,7618 |
| 6 | 505,857 | 505,866 | 110,533 | 107,071 | 224,6619 | 217,1748 |
| 7 | 102,924 | 102,866 | 47,275 | 45,974 | 134,3179 | 133,4362 |
| 8 | 587,956 | 586,477 | 10,206 | 8,93 | 24,0414 | 21,8352 |
| 9 | 365,74 | 364,956 | 16,599 | 15,106 | 33,8702 | 30,9141 |
| 10 | 148,699 | 148,464 | 13,77 | 12,976 | 27,9843 | 26,1933 |
| 11 | 369,001 | 366,265 | 15,784 | 14,822 | 37,4508 | 36,0967 |
| 12 | 232,306 | 230,841 | 24,641 | 23,688 | 50,2218 | 48,4214 |
| 13 | 94,404 | 93,985 | 23,104 | 22,664 | 48,1577 | 47,0064 |
| 14 | 52,926 | 52,5 | 283,562 | 281,695 | 419,6176 | 411,3436 |
| 15 | 57,972 | 57,804 | 282,363 | 280,853 | 412,8216 | 406,113 |
| 16 | 52,423 | 52,168 | 34,583 | 33,869 | 65,8575 | 62,5742 |
| 17 | 5,772 | 5,245 | 96,471 | 94,602 | 70,2832 | 67,1624 |
| 18 | 2,147 | 1,793 | 98,916 | 97,343 | 73,2599 | 70,6706 |
| 19 | 11,12 | 11,093 | 53,011 | 52,356 | 37,2257 | 36,3799 |
| 20 | 9,986 | 9,87 | 123,141 | 121,931 | 97,8849 | 95,1113 |
| 21 | 1,073 | 0,885 | 121,044 | 120,011 | 95,1506 | 92,7414 |

From table 4 it is known that the results of the internal forces (Pu, Mu, and Vu) as a whole show that SNI 1726-2019 is smaller than the results of SNI 1726-2012 in the earthquake spectrum response method. So, from the calculation of internal forces, the structure of this lecture building can be categorized as unable to withstand earthquake forces based on the latest SNI 1726-2019 planning method. So that the structure of this building needs to be given rehabilitation in order to be able to withstand the earthquake loads that will occur, both mild to heavy rehabilitation. This is reinforced by (Novianti, 2015) if the values ​​of Pu, Mu, and Vu at SNI 1726-2012 are lower, it is necessary or not to re-check the buildings that have been built in accordance with SNI 1726-2002.

In addition, in the findings of previous research, many buildings have to be given structural rehabilitation to avoid structural damage because in the planning they still use old methods such as SNI 1726-2002 which later in the SNI 1726-2012 method experienced changes in working methods. additional labor or labor so that the structure is considered unable to withstand the load anymore (Hastono & Syamsudin, 2018; Purnijanto, et al., 2014; Soelarso; 2016).

- Conclusion

Based on the above discussion, it can be concluded that the structure of the lecture building which still uses the SNI 1726-2012 method must be given rehabilitation in order to be able to withstand earthquake loads based on the new SNI 1726-2019 method. This happened because of changes in the earthquake map caused by the discovery of new faults in Indonesia. If the building when planned is given a large safe limit to the seismic force, there is a possibility that if it is reviewed the building is still suitable for use with the new method, but if the building planning provides a small safe limit, then usually if a review is carried out based on the new planning method, certainly it would be recommended that rehabilitation be carried out in order to meet the safety limits of the new method/standard.

- Reference

[1] 	A, S. (2017). Perbandingan Analisis Respons Spektrum dan Analisis Time History Pada Kinerja Struktur Gedung. Universitas Muhammadiyah Sotong.

[2]	Badan Standar Nasional. (2012). Standar Nasional Indonesia 1726-2012. Badan Standar Nasional: Jakarta.

[3]	Badan Standar Nasional. (2019). Standar Nasional Indonesia 1726-2019. Badan Standar Nasional: Jakarta.

[4]	Bayyinah, D. A. L. N., & Faimun. (2017). Studi Perbandingan Analisis Respon Spektra dan Time History untuk Desain Gedung. Jurnal Teknik ITS, 6(1), 33–38. https://doi.org/10.12962/j23373539.v6i1.21617

[5]	Dadawala, S., & N. R., C. (2018). Response spectrum analysis of multi storied buildings: A review. 2018 Advances in Science and Engineering Technology International Conferences, ASET 2018, 1–5. https://doi.org/10.1109/ICASET.2018.8376755

[6]	Dwi Sungkono, K. K. (2019). Respon Spektra Gempa Kota Yogyakarta, Surakarta Dan Semarang Berdasarkan Peta Gempa Sni 2012 Dan Peta Gempa 2017. JUTEKS : Jurnal Teknik Sipil, 4(1), 39–44. https://doi.org/10.32511/juteks.v4i1.304

[7]	Faizah, R. (2015). Studi Perbandingan Pembebanan Gempa Statik Ekuivalen dan Dinamik Time History pada Gedung Bertingkat di Yogyakarta. Jurnal Ilmiah Semesta Teknika, 18(2), 190–199.

[8]	Handayani, L. F. (2015). Dinamis Spektrum Respons Dengan Permodelan Akibat Gempa Berdasarkan Sni 1726-2012.

[9]	Hastono, B., & Syamsudin, R. (2018). Perbandingan Ketahanan Gempa SNI 03-1726-2002 & SNI 03-1726-2012 Pada Perencanaan Bangunan Gedung Di Kota Aceh. Ge-STRAM: Jurnal Perencanaan dan Rekayasa Sipil, 1(1), 1-7

[10]	Novianti, R. (2015). Perbandingan Spektrum Respons Desain SNI 1726:2012 Terhadap SNI 03 1726-2002 Di Berbagai Kota Di Indonesia. Skripsi. Universitas Trisakti

[11]	Pratiwi, G. A., & Widodo. (2012). Analisis Dan Desain Struktur Beton Bertingkat Banyak Berdasarkan Perbandingan Analisis Respons Spektrum Dan Dinamik Riwayat Waktu. 281–293.

[12]	Purnijanto, B., Wiwoho, M., & Crista, N. H. (2014). Perbandingan analisis struktur gedung fakultas psikologi USM (Empat Lantai Gedung T) menggunakan SNI gempa 03-1726-2002 dengan SNI gempa 03-1726-2012. Teknika, 9(2), 12-22.

[13]	Rohman, R. K. (2017). Studi Perbandingan Analisis Gaya Gempa Terhadap Struktur Gedung Di Kota Madiun Berdasar SNI 1726 2002 dan RSNI 201X. Jurnal Agri-Tek, 15(1).

[14]	Safwan, M. (2018). Korelasi Antara Cone Penetration (CPT) dengan Standart Penetration Test (SPT) di Daerah Monumen Jogja Kembali dan Sekitarnya. Universitas Atma Jaya Yogyakarta.

[15]	Soelarso, S., & Baehaki, B. (2016). Analisis Perbandingan Simpangan Horisontal (Drift) Pada Struktur Gedung Tahan Gempa Dengan 3) Menggunakan Pengaku Lateral (Bracing) Berdasarkan Sni 03-1726-2002 Dan Sni 03-1726-2012. Jurnal Fondasi, 5(1).

[16]	Suntoko, H., Marwanto, A., Mp, A., & Iswanto, E. R. (2019). Analisis Spektrum Respon Desain Gedung Reaktor RDE Menggunakan SAP2000. Jurnal Pengembangan Energi Nuklir, 21(1), 1–7.

[17]	W, P. (2015). Perencanaan Struktur Gedung Bank 4 Lantai Tahan Gempa dengan Sistem Daktail Penuh di Wilayah Gempa 3.